Lab 03 Instructions:

- Download code from BbLearn

  - Create Folder of same structure : as explained in the class

- Follow the circuit set-up with two sample acquisition.

- Performing calibration:

  - Potentiometer pendulum is pre-calibrated.

    - Use slope and intercept provide, you collected last week.

  - 4 point calibration for accelerometer

    - Write down mean voltage displayed on the Figure window.

    - Remember to change filename every time for your calibration

- Estimate slope and intercept (Use the Calibration Plotting Code)

- Perform pendulum normal acceleration experiment

  - Check calibration is done properly (need both calibration of potentiometer and accelerometer, you should see same acceleration values as in table for the corresponding orientations )

  - Release from -90 or 90 degs and collect acceleration and pendulum angle data.

  - Check the data file for accuracy

- Post lab

  - Submit Acceleration Calibration Plot

  - Plot of Normal Acceleration

  - Discrete derivative of potentiometer?

  - Accelerometer vs. calculation of acceleration from pendulum angular data?

    - Do they work the same?

    - Noise?